# How the training loss is computed

Gated orientation U-Net for denoising EBSD orientation maps (run blind_both_seed1)

Azza Osman · 29 October 2026

# Overview

Training works by showing the network noisy maps whose clean version we know, measuring how far its output is from the clean map, and adjusting the network to make that distance smaller. The single number that measures the distance is the loss. This document builds it up step by step:

Section 1: the data, the two kinds of noise, and the two masks that record where the noise is.

Section 2: what the network outputs at each pixel: two repair paths and a gate that chooses between them.

Section 3: how the error of one pixel is measured.

Section 4: why some pixels count more than others.

Section 5: the four parts of the loss, and how they are added up.

Sections 6–9: a worked example, why we did not use a plain L1 loss or mean absolute error, implementation details, and a list of all symbols.

In short, the loss is a weighted mean absolute error of the misorientation angle, plus three terms that train the parts of the network directly.

# 1. The data and the two kinds of noise

Each training example starts from a clean map: 128 × 128 pixels, each holding the crystal orientation at that point, stored as a unit quaternion qi⋆ (four numbers that describe a rotation). From it we make a noisy map by adding two kinds of noise, with a strength drawn at random for every map:

Scatter. Every pixel is turned by a small random rotation, with an angle between 0 and a maximum θmax chosen between 1° and 15°. The orientation is slightly off, but still close to the truth.

Misindexing. A fraction p of the pixels, chosen between 1% and 20%, is replaced by a completely random orientation. Grain-boundary pixels are 4 times more likely to be chosen, as in real EBSD. These pixels keep no information about their true orientation.

Because we add the noise ourselves, we know exactly where it is. Two yes/no maps, called masks, record this for every pixel i:

| Mask | Value 1 means | Where it comes from |
| --- | --- | --- |
| bi, misindexing mask | pixel i was misindexed (replaced by a random orientation) | recorded by the noise generator when it replaces the pixel: the exact answer |
| ei, boundary mask | pixel i lies on a grain boundary | the clean map: the pixel's disorientation to one of its 4 neighbours exceeds 0.1° |

The masks exist only because the data are simulated. The network never sees them as input; they are used only to compute the loss. They are recomputed for every batch, after the random rotations and flips used for augmentation, so they always match the map.

Each training step takes a batch of 16 noisy maps (N = 16 × 128 × 128 = 262,144 pixels), computes the loss, and updates the network's weights once. One epoch is 2,437 steps (38,998 training maps ÷ 16 maps per step).

# 2. What the network predicts at each pixel

The two kinds of noise need different repairs. A scattered pixel needs a small correction of the orientation it already has. A misindexed pixel needs a completely new orientation, because its current one is meaningless. The network therefore has two repair paths, and a switch, the gate, that chooses between them at every pixel:

| Output | What it is | Meant for |
| --- | --- | --- |
| Refine path, qiref | The pixel's orientation corrected by a small amount: a weighted average of the neighbouring orientations in a 9 × 9 window (the network chooses the weights), followed by a small learned rotation of at most 25° in total. | Scattered pixels. |
| Replace path, qirep | A completely new orientation, predicted from scratch as 6 numbers and turned into a rotation by Gram–Schmidt orthogonalisation. | Misindexed pixels. |
| Gate, gi | The switch: a number between 0 (use the refine path) and 1 (use the replace path). | Choosing the path, per pixel. |

How the gate works. For every pixel the network outputs a score zi (a logit). The sigmoid σ(zi)=1/(1+e−zi) turns it into a probability, between 0 and 1, that the pixel is misindexed. The gate value gi is this probability with a small dead zone removed:

gi=maxσ(zi)−0.05, 00.95

Below σ(zi) = 0.05 the gate is exactly 0 (closed). A plain sigmoid is never exactly 0, and even a “closed” gate of 0.001 would still move a pixel by a few hundredths of a degree, as large as the errors being corrected. The gate's bias starts at −4, and σ(−4) = 0.018 < 0.05, so every gate starts closed: at the start of training the output is the refine path everywhere.

The final orientation blends the two paths by spherical linear interpolation (slerp), the rotation analogue of a weighted average:

qiout=slerpqiref, qirep, gi

gi = 0 gives the refine path and gi = 1 the replace path. Before blending, qirep is written in the symmetry-equivalent form closest to qiref, so the blend takes the short way.

Gate and mask together. A perfect gate is open (gi = 1) exactly where bi = 1 and closed everywhere else. The misindexing mask is therefore the answer key for the gate, and one part of the loss (Section 5.2) compares the two directly.

# 3. The error of one pixel

## 3.1 The disorientation angle

Nickel has cubic crystal symmetry: 24 rotations leave the crystal lattice unchanged, so the same physical orientation can be written in 24 different ways (and, as a quaternion, with either sign, q or −q). Comparing the stored numbers directly would call a correct prediction wrong. The error is instead the disorientation θ: the smallest rotation angle that turns the predicted orientation into the true one, over all symmetry-equivalent descriptions. It lies between 0° and 62.8°, the largest possible value for cubic symmetry.

With q the prediction and q⋆ the truth, form the relative rotation m=q⋆⊗q−1 and sort the absolute values of its four components, m(1)≥m(2)≥m(3)≥m(4). For cubic symmetry the best match over all symmetry operators has a closed form:

cosθ2=min1, maxm(1), m(1)+m(2)2, m(1)+m(2)+m(3)+m(4)2

The three candidates belong to the identity, the 2- and 4-fold axes, and the 3-fold axes. This gives the same angle as an explicit search over all 24 × 24 symmetry combinations (checked to about 10−7 degrees) and is about 67 times faster. The evaluation (the ALL, BOUNDARY and CORRUPTED errors) uses the same angle, so the network is trained and judged on the same physical quantity.

## 3.2 The per-pixel loss

The angle is turned into the loss of one pixel, ℓ:

ℓ(q,q⋆)=1−cosθ2+ε−ε,ε=10−10

Using the identity 1−cosx=2sin2(x/2):

1−cosθ2=2sin2θ4⇒ℓ≈2sinθ4≈θ22

with θ in radians. In words, ℓ is the angle itself divided by the constant 22 ≈ 2.83. The small constant ε only matters below

θ0=22ε=2.8×10−5 rad=0.0016∘

where ℓ turns smoothly quadratic instead of having the sharp corner that |θ| has at zero. That is about 40 times below the smallest mean error the model reaches on the test sets (0.06° at 1° scatter), so over the whole working range ℓ behaves like the angle:

| Error θ | Loss ℓ | θ/(22) | Ratio |
| --- | --- | --- | --- |
| 0.01° | 5.25 × 10⁻⁵ | 6.17 × 10⁻⁵ | 0.851 |
| 0.1° | 6.07 × 10⁻⁴ | 6.17 × 10⁻⁴ | 0.984 |
| 0.5° | 0.00308 | 0.00309 | 0.997 |
| 1° | 0.00616 | 0.00617 | 0.998 |
| 5° | 0.0308 | 0.0309 | 1.000 |
| 10° | 0.0617 | 0.0617 | 1.000 |
| 40° | 0.246 | 0.247 | 0.995 |
| 62.8° | 0.383 | 0.388 | 0.988 |

## 3.3 Why the loss uses cos(θ/2) instead of θ

The obvious choice, θ itself, is computed as θ=2arccosκ with κ=cos(θ/2). Its derivative

dθdκ=−21−κ2→−∞asκ→1

is infinite exactly at zero error, the point every pixel is being pushed towards. In 32-bit floating point the problem is concrete: κ rounds to exactly 1 for every error below 0.028°, the derivative becomes infinite and the update turns into NaN (not a number), which stops training just as it starts to succeed. Errors of a few hundredths of a degree are common once the model works. The derivative of ℓ stays finite:

dℓdκ=−121−κ+ε,dℓdκ≤12ε=5×104

Measured per degree of error it is constant, about 0.0062 per degree from 0.1° to 10° and beyond, exactly like an L1 loss. The angle θ itself is computed only for reporting.

# 4. Pixel weights

Using the two masks of Section 1, each pixel gets a weight:

ωi=1+5bi+2ei

A pixel therefore weighs 1 (interior, scatter only), 3 (grain boundary), 6 (misindexed) or 8 (misindexed and on a boundary). Measured on 100 validation maps, this shifts the attention as follows:

| Pixel group | ωi | Share of pixels | Share of the total weight |
| --- | --- | --- | --- |
| Interior, scatter only | 1 | 77.0% | 42.3% |
| Grain boundary, not misindexed | 3 | 12.3% | 20.7% |
| Misindexed (interior or boundary) | 6 or 8 | 10.7% | 37.1% |

Why: in an unweighted mean, 77% of the loss would come from the easy interior pixels, while the two groups that matter most scientifically, and where filters fail (grain boundaries and misindexed points), would be a small minority. They are also exactly the BOUNDARY and CORRUPTED columns of the evaluation.

# 5. The four parts of the loss

The loss checks the final map and, separately, each of the three outputs of Section 2. All sums run over the N = 262,144 pixels of one batch (the pixels of all 16 maps are pooled), and qi⋆ is the true orientation of pixel i.

## 5.1 Main part: is the final map right?

ℒmain=iωiℓqiout,qi⋆iωi

The weighted mean error of the final orientation qiout, the map the user actually receives. Dividing by iωi keeps it a weighted average, so its size does not depend on how many misindexed pixels a batch contains (1–20% per map). It trains every part of the network the output depends on: the refine path where gi < 1, the replace path where gi > 0, and the gate where it is open.

## 5.2 Gate part: does the gate open exactly on the misindexed pixels?

ℒgate=−1Ni=1Nbilogσ(zi)+(1−bi)log1−σ(zi)

The binary cross-entropy between the gate's probability σ(zi) and the misindexing mask bi: the cost is small when the gate says “misindexed” on a misindexed pixel, or “fine” on a fine one, and large otherwise. It is averaged over all N pixels with no extra class weights, and computed from the score zi, before the dead zone, in the numerically stable form (PyTorch binary_cross_entropy_with_logits). Examples of the cost of one pixel:

| Mask bi | Gate probability σ(zi) | Cost |
| --- | --- | --- |
| 1 (misindexed) | 0.98 | 0.020 |
| 1 (misindexed) | 0.60 | 0.511 |
| 0 (not misindexed) | 0.01 | 0.010 |
| 0 (not misindexed) | 0.30 | 0.357 |

Why it is needed. Because of the dead zone, a closed gate (σ(zi) < 0.05) passes no gradient back from ℒmain at all, and every gate starts closed. Trained through ℒmain alone, the gate could never learn to open. The mask gives it the answer directly. At evaluation a pixel counts as flagged when gi > 0.5; at the saved epoch (59) the gate's precision is 99.8% (flagged pixels that really are misindexed) and its recall 98.7% (misindexed pixels that are flagged), on the validation maps.

## 5.3 Refine part: is the refine path right on the pixels it is meant for?

ℒrefine=1N0i:bi=0ℓqiref,qi⋆

N0 is the number of pixels that were not misindexed (bi = 0). The term grades the refine path on its own, before any blending, on the 80–99% of pixels it is responsible for. It trains the refine path directly, including the network-chosen neighbour weights, instead of only through the blended output. These pixels decide most of the mean error ALL, and the term carries the largest weight (5).

## 5.4 Replace part: is the replace path right on the pixels it is meant for?

ℒreplace=1N1i:bi=1ℓqirep,qi⋆

N1 is the number of misindexed pixels (bi = 1). Why it is needed. At the start every gate is closed, so the output equals the refine path and ℒmain gives the replace path no gradient. Without this term the replace path would never improve, and the gate would have no reason to open: a chicken-and-egg problem. Grading the replace path directly on the misindexed pixels from the first step breaks it. (If a batch had no misindexed pixel the term would be 0; in training this cannot happen, since every map has at least 1% misindexed pixels.)

## 5.5 Adding the four parts

ℒ=ℒmain+1·ℒgate+5·ℒrefine+0.5·ℒreplace

Each part answers one question:

ℒmain: how wrong is the final map? Misindexed and boundary pixels count more.

ℒgate: does the gate open exactly where the misindexing mask is 1?

ℒrefine: how wrong is the refine path on the pixels that were not misindexed?

ℒreplace: how wrong is the replace path on the misindexed pixels?

The weights 1, 1, 5 and 0.5 are set in the training configuration. The table gives the values on the 500 validation maps at epoch 59, the saved model (best.pth):

| Part | Value | Weight | Contribution | Share | In degrees |
| --- | --- | --- | --- | --- | --- |
| ℒmain | 0.00897 | 1 | 0.00897 | 21.9% | 1.45° (weighted mean error of the output) |
| ℒgate | 0.00773 | 1 | 0.00773 | 18.9% | – |
| ℒrefine | 0.00330 | 5 | 0.01648 | 40.2% | 0.53° (refine path, non-misindexed pixels) |
| ℒreplace | 0.01554 | 0.5 | 0.00777 | 19.0% | 2.52° (replace path, misindexed pixels) |
| Total |  |  | 0.04094 | 100% |  |

The degrees use θ≈22ℓ. As a check, the replace value (2.52°) agrees with the separately measured mean error of the final output on misindexed pixels, CORRUPTED = 2.49°: the loss is the angle in disguise. The logged total, 0.04094, matches the sum of the four contributions.

Which parts of the network each part of the loss trains:

| Loss part | Refine path | Replace path | Gate | Shared U-Net |
| --- | --- | --- | --- | --- |
| ℒmain | yes, where gi < 1 | yes, where gi > 0 | only where open | yes |
| ℒgate | – | – | yes, every pixel | yes |
| ℒrefine | yes | – | – | yes |
| ℒreplace | – | yes | – | yes |

# 6. Worked example with four pixels

Suppose a batch had only these four pixels:

| Pixel | θ | bi | ei | ωi | ℓ | ωiℓ |
| --- | --- | --- | --- | --- | --- | --- |
| Interior, scatter only | 0.5° | 0 | 0 | 1 | 0.00308 | 0.00308 |
| On a grain boundary | 2° | 0 | 1 | 3 | 0.01233 | 0.03699 |
| Misindexed, interior | 3° | 1 | 0 | 6 | 0.01850 | 0.11101 |
| Misindexed, on a boundary | 5° | 1 | 1 | 8 | 0.03084 | 0.24673 |
| Sum |  |  |  | 18 | 0.06475 | 0.39781 |

ℒmain = 0.39781 / 18 = 0.0221, against an unweighted mean of 0.0162. The two misindexed pixels hold 14 of the 18 weight units (78%), so the network is pushed hardest where the noise is worst. In the same example, ℒrefine would average the refine-path errors of pixels 1 and 2, and ℒreplace the replace-path errors of pixels 3 and 4.

# 7. Why not a plain L1 loss or mean absolute error?

“L1 loss” can mean three different things here. The short answer: our loss is an L1 / MAE loss, on the right quantity (the disorientation angle), written so that its gradient stays finite, and weighted.

## 7.1 L1 on the stored numbers (Euler angles or quaternion components)

This is not a valid measure of orientation error: it penalises correct predictions.

| Prediction vs truth | L1 of the numbers | True error |
| --- | --- | --- |
| Euler (0°, 0°, 0°) vs (90°, 0°, 0°) | 90° | 0°: a 90° turn about [001] is a symmetry of the cubic lattice |
| Euler (359°, 0°, 0°) vs (1°, 0°, 0°) | 358° | 2°: angles wrap around at 360° |
| Euler (30°, 0°, 60°) vs (60°, 0°, 30°) | 60° | 0°: when Φ = 0 only φ₁ + φ₂ matters |
| Quaternion q vs −q | at least 2 | 0°: q and −q are the same rotation |

A network trained like this receives contradictory targets for the same physical orientation.

## 7.2 MAE of the disorientation angle

This is the right quantity, and it is what we optimise: since ℓ≈θ/(22), minimising the weighted mean of ℓ is minimising the weighted mean of θ, up to a constant factor. It is also how the model is selected: best.pth is the epoch with the lowest validation mean error ALL, in degrees. The only differences from writing |θ| directly are deliberate: the loss is computed through cos(θ/2), which keeps the gradient finite (Section 3.3), and the corner of |θ| at zero is rounded off below 0.0016°.

## 7.3 Unweighted MAE

If every pixel weighed the same, 77% of the loss would sit on interior pixels that only carry scatter and 23% on misindexed and boundary pixels. The weights ωi raise the share of those two groups to 58% (Section 4), and the gate, refine and replace parts add direct supervision for each part of the network.

## 7.4 And why not L2 (mean squared error)?

The squared alternative, also available in the code (loss: cosine), is

1−cosθ2≈θ28

Its gradient grows with the error: a 0.1° error is pushed 100 times less than a 10° error (7.6×10−6 against 7.6×10−4 per degree), while ℓ pushes both equally (6.2×10−3 per degree). An L2 loss therefore stops refining the small scatter errors that make up most pixels, lets a few large errors dominate, and, as is well known in image denoising, tends to blur sharp edges, here the grain boundaries. The L1-like ℓ keeps the same push on every error and preserves boundaries.

Summary of the alternatives:

| Candidate | Problem | What our loss does instead |
| --- | --- | --- |
| L1 / MAE on Euler angles or quaternion numbers | Ignores crystal symmetry, the double cover (q = −q) and angle wrap-around; penalises correct answers | Uses the disorientation angle θ (symmetry-aware) |
| MAE written as θ=2arccos(·) | Infinite derivative at zero error; NaN in float32 below 0.028° | Same quantity through ℓ≈θ/(22) |
| Unweighted MAE | 77% of the loss on easy interior pixels | Weights ωi=1+5bi+2ei |
| L2 / MSE | Small errors neglected, large errors dominate, boundaries blur | L1-like ℓ |
| Main part only | The gate cannot open (dead zone); the replace path never trains | Gate, refine and replace parts |

# 8. Implementation details

Code: architecture.py (denoise_loss, boundary_mask), orientation_torch.py (cos_half_disorientation, disorientation_loss) and train.py (training loop), all in MODEL_PIPELINE/src.

Precision: the U-Net layers run in bfloat16; all orientation maths (quaternion products, Gram–Schmidt, slerp) and the loss run in float32.

Optimiser: AdamW, learning rate 2×10−4 decayed by a cosine schedule to 2×10−6, no weight decay, 60 epochs of 2,437 steps, gradient norm clipped at 1.0.

Logged values: the training loss is the average of the 2,437 batch losses of one epoch (augmented maps, fresh noise, and a network that keeps changing during the epoch); the validation loss is the same formula on the 500 validation maps with fixed noise and no augmentation, at the end of the epoch.

Settings (training configuration blind_both_seed1.yaml, plus two constants in the code):

| Setting | Value | Meaning |
| --- | --- | --- |
| loss | charbonnier | per-pixel loss ℓ (Section 3.2) |
| loss_eps | 1e-10 | ε in ℓ |
| w_bad | 5 | extra weight of a misindexed pixel in ωi |
| w_bnd | 2 | extra weight of a boundary pixel in ωi |
| w_gate | 1 | weight of ℒgate |
| w_refine | 5 | weight of ℒrefine |
| w_replace | 0.5 | weight of ℒreplace |
| max_refine_deg | 25 | largest rotation the refine path may add (degrees) |
| gate dead zone (code) | 0.05 | σ(z) below which the gate is exactly closed |
| initial gate bias (code) | −4 | every gate starts closed (σ(−4) = 0.018) |

# 9. Symbols

| Symbol | Meaning | Section |
| --- | --- | --- |
| i, N | a pixel; the number of pixels in one batch (262,144) | 1 |
| qi⋆ | true (clean) orientation of pixel i | 1 |
| θmax, p | scatter strength (1–15°) and misindexed fraction (1–20%) of a map | 1 |
| bi | misindexing mask: 1 if pixel i was replaced by a random orientation | 1 |
| ei | boundary mask: 1 if pixel i lies on a grain boundary | 1 |
| qiref, qirep | orientation from the refine path and from the replace path | 2 |
| zi, σ(zi), gi | gate score, gate probability, gate value used for blending | 2 |
| qiout | final orientation, the blend of the two paths | 2 |
| θ | disorientation angle between a prediction and the truth | 3.1 |
| ℓ | loss of one pixel, about θ/(22) | 3.2 |
| ε | small constant in ℓ, 10−10 | 3.2 |
| ωi | pixel weight, 1+5bi+2ei | 4 |
| N0, N1 | number of non-misindexed and of misindexed pixels in the batch | 5 |
| ℒ | total loss, the number training makes smaller | 5.5 |